Coquet Hydro Ltd R&D

Retrospective Review of the Guyzance Hydroelectric Projects R&D Qualities

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Projects that count as R&D must show:

1 – How has the project looked for an advance in science and technology

2 – How has the project had to overcome uncertainty

3 – How has the project tried to overcome this uncertainty

4 – How has the project overcome this uncertainty which a professional in the field could not easily have worked out

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

1 – Advance in the field

Aim to create an advance in the overall field, not just your business.

#DRAFT1

By proving that low-head hydroelectric generation on meander cut-offs is technically feasible, a precedent could be set for further development of this resource nationally.

#DRAFT2

By proving that low-head hydroelectric generation (using an Archimedes screw) on a meander cut-off is technically feasible and provides clear aquatic-ecological benefits, a precedent has been set for further development of the hydroelectric resource nationally within this niche geographical feature.

Can’t just be an existing technology that has been used for the first time in your sector.

#DRAFT1

What is novel about this project is not that gravity has been used to yield an electrical output (Archimedes screw utilised in a meander cut-off), it is the fact that this new interface could provide a more sustainable method of electricity production in a limited range of geographical locations when compared to contemporary schemes using the same technology.

#DRAFT2

Although Archimedes screw systems have been deployed in low-head hydroelectric formats across the UK, the vast majority capitalise off of the artificial infrastructure facilitating their elevation head (e.g weirs). Coquet Hydro is pioneering a low-head hydroelectric system which obtains a natural elevation head by making use of a meander cut-off. Resultantly, this provides both a sustainable technical working principle, as well as an ecologically suitable system fit for the migratory routes of salmonids in its waters.

2 – Show there was uncertainty

A scientific or technological uncertainty exists when an expert on the subject cannot say if something is technologically possible or how it can be done – even after referring to all available evidence.

#DRAFT1

Uncertainty lies in whether or not this system is more or less of an obstruction to fish passing along the watercourse than similar schemes employing Archimedes screw.

#DRAFT2

Uncertainty lies in the potential aquatic-ecological benefits possessed by the meander cut-off low-head hydroelectric scheme. Hence the equivalent hypothesis to be tested by this R&D project is “Low-head meander cross-cut hydroelectric systems have lower impact on fish migration than weir-based alternatives”.

This means that your company or experts in the field cannot already know about the advance or the way you could achieve it.

#DRAFT1

Due to the novelty of the project, neither Coquet Hydro Ltd nor any experts in the field know the exact impact on fish migration a meander cut-off hydroelectric system will have. Only educated guesses as to possible influences may be deduced by cross-examination of tributaries effects on migration along watercourses.

3 – Explain how you tried to overcome the uncertainty

You should show that the R&D needed research, testing and analysis to develop it.

To overcome this uncertainty the Guyzance Hydroelectric project was developed on the river Coquet by Coquet Hydro Ltd. This provided both a hydroelectric plant for the production of renewable power, as well as a testbed for research into the aforementioned hypothesis. This infrastructure facilitated testing of salmon presence in the outlet basin, thus indicating the abatement time caused by the flow.

You need to be able to explain the work you did to overcome the uncertainty. This can be a simple description of the successes and failures you had during the project.

Dependent upon project being undertaken.

4 – Show that a professional in the field could not work this out

You should explain why a professional could not easily work out your advance

You can do this by showing that other attempts to find a solution failed

You can also show that the people working on your project are professionals in that field and get them to explain the uncertainties involved